Smart Orchid Greenhouse

Step-by-Step Build Guide

For young builders and their grown-up helpers

Phalaenopsis orchid • IKEA ÅKERBÄR greenhouse • ESP32-S2 Feather • Raspberry Pi 4

TIP How to use this guide Do the parts in order. Do not skip ahead. Each part ends with a small test, so you always know that everything works before you move on. Look for these colored boxes: Red = STOP AND READ. Something important that can break a part if you get it wrong. Orange = GROWN-UP HELPER. Ask an adult to do this step or to watch you do it. Green = WHAT YOU SHOULD SEE. How to know the step worked. Blue = TIP. Helpful extra information.

# What's Inside This Guide

| Part | What you will do | Who |
| --- | --- | --- |
| 1 | Understand what we are building | Everyone |
| 2 | Collect and check all the parts | Young builder |
| 3 | The 6 golden rules | Everyone |
| 4 | Set up the Raspberry Pi 4 (install its system on the memory card) | Grown-up + young builder |
| 5 | Get to know the ESP32-S2 Feather board | Young builder |
| 6 | Put the test program on the Feather | Young builder + grown-up |
| 7 | Connect the sensors one at a time and test each one | Young builder |
| 8 | If something does not work | Everyone |
| 9 | Put everything in the greenhouse | Young builder + grown-up |
| 10 | Add the battery (last!) | Grown-up |
| 11 | What happens next | Everyone |
| 12 | Final checklist | Everyone |

# Part 1. What Are We Building?

We are building a little weather station for an orchid. It will watch the orchid's home all day and all night and remember everything it measures.

Think of it like a body:

- The senses are three small sensor boards. One feels the air (how warm and how damp), one feels the soil (wet or dry), and one sees the light (bright or dark).

- The messenger is a small computer board called the ESP32-S2 Feather. It lives outside on the veranda next to the greenhouse. It asks the sensors "What do you feel?" and sends the answers through Wi-Fi.

- The brain is the Raspberry Pi 4. It stays inside the house. It receives the messages, saves them, and later makes charts so we can see how the orchid is doing.

TIP Why does the orchid need this? A Phalaenopsis orchid often starts making flowers after a few weeks of cool nights (about 13–16 °C). The Raspberry Pi will count those cool nights for us, so we know if the orchid is getting what it needs.

# Part 2. Collect and Check All the Parts

Put everything on a clean, dry table. Do not put anything in the greenhouse yet. Check off each part when you find it.

## The main parts

| ☐ | Part name | Nickname | What it does |
| --- | --- | --- | --- |
| ☐ | Adafruit ESP32-S2 Feather | The Feather | The messenger. Reads the sensors and sends data by Wi-Fi. |
| ☐ | Adafruit BH1750 Light Sensor – STEMMA QT (PID 4681) | Light sensor | Measures how bright the light is. Has two small white plugs. |
| ☐ | Adafruit BME280 (STEMMA QT) | Air sensor | Measures temperature, humidity (how damp the air is), and air pressure. Has two small white plugs. |
| ☐ | Adafruit STEMMA Soil Sensor | Soil sensor | A long fork-shaped board that goes into the pot. Measures how wet the soil is. Has one white plug. |
| ☐ | Raspberry Pi 4 | The Pi | The brain. Lives inside the house. |
| ☐ | Pimoroni Inky pHAT | The screen | A small e-ink screen that sits on top of the Pi and will show the readings. |

## The cables

| ☐ | Cable | Nickname | Where it goes |
| --- | --- | --- | --- |
| ☐ | STEMMA QT cable, 300 mm (PID 5384) | Long cable | Feather → light sensor |
| ☐ | STEMMA QT cable, 200 mm (PID 4401), 2 pieces | Medium cable | Light sensor → air sensor. The second one is a spare. |
| ☐ | JST PH → JST SH cable, 200 mm | Soil cable | Air sensor → soil sensor. The only cable with a tiny plug on one end and a bigger plug on the other end. |
| ☐ | STEMMA QT cable, 100 mm (PID 4210) | Short cable | Spare. Also useful for testing the air sensor before the light sensor arrives. |
| ☐ | USB-C cable | USB cable | Connects the Feather to the computer. Must be a DATA cable (see the red box below). |

TIP Good news: no soldering, no loose wires. All the STEMMA QT cables have the same small plug on both ends, so it does not matter which end goes where. Only the soil cable has two different plugs. You do not need: the loose metal pins that came in a bag with the Feather, the old HiLetgo GY-302 light sensor, or the colorful jumper wires. Put them away.

## Other things you need

☐  A Mac computer (the grown-up's computer is fine).

☐  The Raspberry Pi 4 with its memory card and power supply.

☐  The LiPo battery. Keep it in a drawer for now. We use it only in Part 10.

☐  A small waterproof box for the Feather and battery (needed only in Part 9).

☐  Sticky notes and a pen for labels.

STOP AND READ Not every USB-C cable can carry data. Some cables (often the ones that come with toys, lamps or power banks) can only charge. If you use one of those, the Mac will never see the Feather, and it will look like the board is broken. If something "does not show up" later, the cable is the first thing to swap.

# Part 3. The 6 Golden Rules

Read these out loud together before you start.

- No battery until Part 10. The Feather gets its power from the USB cable while we build and test.

- Unplug the USB cable before you plug or unplug any sensor or cable. Then plug USB back in to test.

- Never force a plug. The small plugs only go in one way. If it does not slide in gently, turn it around and look again.

- Hold the plug, not the wire. When you unplug a cable, pull on the little white plug, never on the thin wires.

- Keep water away. Do not put the soil sensor in the orchid pot until Part 9.

- If a part looks different from this guide, STOP. Take a clear photo of it and ask before plugging anything in.

# Part 4. Set Up the Raspberry Pi 4

A brand new Raspberry Pi is like a new computer with no system on it. Its "hard drive" is the little microSD card. In this part we copy the Raspberry Pi system onto the card. This is called flashing the card.

GROWN-UP HELPER Do this part together. It takes about 30–45 minutes, mostly waiting. Flashing erases everything on the card. If the card already has another project on it (for example, a game), use a new, empty card instead.

## 4.1 Write down your settings first

Fill in this little table. You will type these into the computer in a minute. Keep the paper somewhere safe.

| Setting | Example | Ours |
| --- | --- | --- |
| Pi name (hostname) | orchid-pi |  |
| Username | orchid |  |
| Password | (make up a good one) |  |
| Home Wi-Fi name | MyHomeWiFi |  |
| Home Wi-Fi password | (from the router sticker) |  |

STOP AND READ The Feather can only use 2.4 GHz Wi-Fi, not 5 GHz. Many home routers have both, sometimes with two names (for example "MyHome" and "MyHome-5G"). Use the 2.4 GHz network (the one without "5G") for both the Pi and, later, the Feather. If you are not sure, the grown-up can check the router settings.

## 4.2 Install Raspberry Pi Imager on the Mac

- On the Mac, open a web browser and go to raspberrypi.com/software.

- Click Download for macOS.

- Open the downloaded file and drag Raspberry Pi Imager into the Applications folder.

- Open Raspberry Pi Imager from Applications. If the Mac asks "Are you sure you want to open it?", click Open.

## 4.3 Flash the microSD card

TIP The Imager program is updated from time to time, so the buttons may look a little different. The choices are always the same: which Pi → which system → which card → your settings → write.

- Put the microSD card into the card reader and plug it into the Mac.

- In Imager, choose the device: Raspberry Pi 4.

- Choose the operating system: Raspberry Pi OS (64-bit) (the one marked "recommended").

- Choose the storage: your microSD card. Check the size! It should match your card (for example 32 GB). Never pick the Mac's own drive.

- Click Next. Imager asks if you want to customize the settings. Say yes (it may be called Edit Settings or it may walk you through several screens).

- Type in the settings from your table: hostname, username and password, Wi-Fi name and password. Set Wi-Fi country to your country, and the correct time zone and keyboard.

- Find the setting called SSH (sometimes under "Services" or "Remote access"). Turn it on and choose Use password authentication. This lets us talk to the Pi from the Mac later.

- Click Save, then Write (or Yes). Imager warns that the card will be erased. Click Yes.

- The Mac may ask for its own password. The grown-up types it.

- Wait. First Imager writes, then it checks ("verifying"). This can take 10–20 minutes.

- When it says the write was successful, take the card out of the Mac.

## 4.4 Start the Pi for the first time

- Make sure the Pi is unplugged.

- Turn the Pi upside down. The microSD slot is on the bottom, at the short end. Slide the card in with the label facing away from the board. It clicks in gently; it sticks out a little. That is normal.

- Optional: if you want to see the Pi's screen, connect a monitor or touchscreen, a keyboard and a mouse. (The Pi 4 uses a micro-HDMI plug, which is smaller than a normal HDMI plug.) You can also skip this and control the Pi from the Mac (step 4.5).

- Plug in the Pi power supply.

- Wait about 5 minutes. The first start is slow because the Pi sets itself up. It may restart by itself once. Do not unplug it during this time.

WHAT YOU SHOULD SEE A red light that stays on (power is OK) and a green light that blinks (the Pi is reading the card). If a screen is connected: after a few minutes you see the Raspberry Pi desktop.

## 4.5 Talk to the Pi from the Mac

This is how we control the Pi without a screen. We type commands in a program called Terminal.

- On the Mac, open Terminal (press Command + Space, type Terminal, press Enter).

- Type this (use your own username and Pi name from the table) and press Enter:

ssh orchid@orchid-pi.local

- The first time, it asks "Are you sure you want to continue connecting?" Type yes and press Enter.

- Type the Pi password and press Enter. You will not see any letters or dots while you type the password. This is normal. Just type it and press Enter.

WHAT YOU SHOULD SEE The line in Terminal changes to something like orchid@orchid-pi:~ $. You are now "inside" the Pi. Everything you type now happens on the Pi, not on the Mac.

## 4.6 Update the Pi

Just like a phone, the Pi needs updates. Type these lines one at a time and press Enter after each. The Pi may ask for the password again.

sudo apt update sudo apt full-upgrade -y

This can take 5–20 minutes. Lots of text will scroll by. Wait until you see the $ sign again. Then restart the Pi:

sudo reboot

Wait 1–2 minutes, then connect again with the ssh line from step 4.5.

## 4.7 Write down the Pi's address

Later, the Feather needs to know where to send its messages. Type:

hostname -I

You will see a number like 192.168.1.42. Write the first number down here: ______________________

STOP AND READ How to turn the Pi off safely: never just pull the plug while it is running. Type sudo shutdown now, wait until the green light stops blinking (about 20 seconds), and only then unplug it.

WHAT YOU SHOULD SEE Part 4 is done when: the Pi starts, the Mac can connect to it with ssh, it is updated, and you wrote down its address.

# Part 5. Get to Know the Feather

Hold the Feather board and find these things. Point to each one and check it off.

☐  USB-C port — the metal plug at one short end. The USB cable goes here.

☐  STEMMA QT port — a tiny white plug with 4 little metal contacts. The long cable goes here.

☐  Reset button — one of two small square buttons. Look for the tiny white word Reset (or RST) next to it. The other button says Boot (or D0) — we do not use that one.

☐  Battery plug — a white plug with 2 contacts, labeled BAT or with + and −. We do NOT use it until Part 10.

☐  Rows of small holes along the long edges — these are for other projects. We do not use them. They stay empty, and that is completely fine.

TIP The letters on the board are very small. A magnifying glass helps, or take a photo with a phone and zoom in.

# Part 6. Put the Test Program on the Feather

The Feather needs a program to know how to read the sensors. We will use CircuitPython, which makes the Feather look like a USB stick on the Mac. You just copy files onto it.

GROWN-UP HELPER The downloads in this part are best done by the grown-up. The young builder can do the copying.

## 6.1 Install CircuitPython on the Feather

There are two ways. Way 1 is the easiest if it works on your Mac. If the Mac shows errors, use Way 2 — it always works.

STOP AND READ Pick the right board. On circuitpython.org there are several boards with similar names. Choose exactly "Feather ESP32-S2" by Adafruit. Do not choose "Feather ESP32-S2 with BME280", "Feather ESP32-S2 TFT" or "Reverse TFT". Those are different boards.

Way 1: drag and drop

- Go to circuitpython.org/board/adafruit_feather_esp32s2/ and download the newest stable version as a .UF2 file. Write down the version number (for example 10.3.1): ____________

- Connect the Feather to the Mac with the USB-C data cable. No sensors attached.

- Press the Reset button two times quickly (click-click, like a double-click on a mouse).

- A new drive appears in Finder, called FTHRS2BOOT.

- Drag the .UF2 file onto FTHRS2BOOT.

- Wait 20 seconds. FTHRS2BOOT disappears and a new drive called CIRCUITPY appears.

TIP A message saying "Disk Not Ejected Properly" is normal. The Feather restarts as soon as it gets the file, so the Mac "loses" the drive. Just close the message. If the Mac says "you don't have permission", or FTHRS2BOOT comes back instead of CIRCUITPY, or all the lights on the board go dark — do not worry. The board is fine. Use Way 2.

Way 2: install from the web browser (always works)

- Open Google Chrome (Safari does not work for this).

- Go to circuitpython.org/board/adafruit_feather_esp32s2/

- Click Open Installer and choose the full CircuitPython install.

- When the installer asks you to put the board in a special mode: hold the Boot button, press and release Reset, then let go of Boot.

- In the Chrome pop-up, pick the device (ESP32-S2 or usbmodem) and click Connect. Say yes if it asks to erase the board.

- Wait and do not unplug the cable. Follow the messages on the screen.

- When CIRCUITPY appears in Finder, the job is done — even if the installer window seems stuck on "Flashing…". Just close the Chrome tab.

WHAT YOU SHOULD SEE A drive named CIRCUITPY on the Mac. Inside there is a file called code.py and a folder called lib. That means CircuitPython is installed!

## 6.2 Add the sensor "libraries"

A library is a ready-made set of instructions that teaches the Feather how to talk to one type of sensor. We need a few of them.

- Go to circuitpython.org/libraries.

- Download the Bundle whose number matches your CircuitPython version (for 10.3.1 that is "Bundle for Version 10.x"). It is a .zip file.

- Double-click the .zip file to open it. Inside, open the folder called lib.

- Copy these 5 things from the bundle's lib folder into the lib folder on the CIRCUITPY drive:

| ☐ | Copy this | It is a... | It is for |
| --- | --- | --- | --- |
| ☐ | adafruit_bme280 | folder | Air sensor |
| ☐ | adafruit_seesaw | folder | Soil sensor |
| ☐ | adafruit_bh1750.mpy | file | Light sensor |
| ☐ | adafruit_bus_device | folder | Helper that all sensors need |
| ☐ | adafruit_register | folder | Helper for the light sensor |

TIP If the Mac says "you don't have permission" while copying, copy with Terminal instead. Type cp -X -R (with a space at the end), drag the folder from Finder into the Terminal window, then type /Volumes/CIRCUITPY/lib/ and press Enter. For example: cp -X -R ~/Downloads/adafruit-circuitpython-bundle-10.x/lib/adafruit_bme280 /Volumes/CIRCUITPY/lib/

## 6.3 Install Thonny (to see what the Feather says)

Thonny is a free, simple program for writing Python. It is installed on the Mac. We use it to edit the test program and to read the messages the Feather prints.

- Go to thonny.org and download Thonny for Mac. Install it like any other app (drag it into Applications).

- Connect the Feather to the Mac. Open Thonny.

- In the menu, click Run → Configure interpreter… (in some versions: Tools → Options → Interpreter).

- In the first list, choose CircuitPython (generic).

- In the "Port" list, choose the one with usbmodem in its name. Click OK.

- If the Mac asks "Allow accessory to connect?", click Allow.

## 6.4 Copy the test program

STOP AND READ The program runs on the Feather, not on the Mac. You do not press "Run" in PyCharm or any other program on the Mac. You just save the program as code.py on the CIRCUITPY drive, and the Feather starts it by itself. If you see ModuleNotFoundError: No module named 'board', the program was started on the Mac by mistake. The code is fine — save it to CIRCUITPY instead.

- In Thonny, click File → Open. If it asks where, choose CircuitPython device. Open code.py.

- Delete everything inside it (Command + A, then Delete).

- Type or copy the program below exactly. Spaces at the beginning of lines matter in Python! Lines starting with # are notes for humans; the Feather ignores them.

- Click File → Save (Command + S). The Feather restarts the program by itself every time you save.

# Orchid greenhouse - sensor test # Checks all 3 sensors every 3 seconds and prints what it finds. import time import board from adafruit_bme280 import basic as adafruit_bme280 from adafruit_seesaw.seesaw import Seesaw import adafruit_bh1750 i2c = board.STEMMA_I2C() # the "talking line" for all sensors air = None soil = None light = None while True: print("------------------------------") # Who is connected? Ask the talking line. while not i2c.try_lock(): pass found = i2c.scan() i2c.unlock() print("Devices found:", [hex(a) for a in found]) # AIR sensor (BME280) - address 0x77 try: if air is None: air = adafruit_bme280.Adafruit_BME280_I2C(i2c) print("AIR: OK temp =", round(air.temperature, 1), "C", " humidity =", round(air.relative_humidity), "%") except Exception: air = None print("AIR: not found") # SOIL sensor - address 0x36 try: if soil is None: soil = Seesaw(i2c, addr=0x36) print("SOIL: OK moisture =", soil.moisture_read()) except Exception: soil = None print("SOIL: not found") # LIGHT sensor (BH1750) - address 0x23 try: if light is None: light = adafruit_bh1750.BH1750(i2c) print("LIGHT: OK light =", round(light.lux), "lux") except Exception: light = None print("LIGHT: not found") time.sleep(3)

WHAT YOU SHOULD SEE At the bottom of Thonny there is a window called Shell. Every 3 seconds it prints something like: Devices found: [] AIR: not found SOIL: not found LIGHT: not found That is correct for now — no sensors are plugged in yet! In Part 7 the "not found" lines will turn into "OK" one by one. If you see a red error with the word ImportError or No module named, one of the libraries from step 6.2 is missing. Check the lib folder again.

# Part 7. Connect the Sensors, One at a Time

All the sensors are joined in one chain, like train cars. Each cable goes from one board to the next one. Here is how it looks at the end:

TIP Why can all three sensors share one chain? They all use a way of talking called I²C (say "eye-squared-see"). It is like a group chat: everyone is in the same chat, but each sensor has its own name, called an address. The Feather calls a sensor by its address, and only that sensor answers. Our addresses: Light sensor = 0x23, Soil sensor = 0x36, Air sensor = 0x77. No two are the same, so there is no mix-up.

TIP Light sensor not here yet? You can start without it. Connect the air sensor straight to the Feather with the short cable (100 mm), then do Step B and Step C. Do Step A when the light sensor arrives (Step A explains how to change the cables).

## Step A. The light sensor (BH1750)

- Unplug the USB cable from the Feather.

- If the short cable (100 mm) is connected from the Feather to the air sensor, take it out at both ends and put it away as a spare.

- Take the long cable (300 mm). Push one end into the STEMMA QT port on the Feather. Push gently until it is fully in.

- Push the other end into either of the two plugs on the light sensor. Keep the other plug free for Step B.

- Place the light sensor with the small chip facing up.

- Plug the USB cable back in. Look at the Shell window in Thonny.

WHAT YOU SHOULD SEE Devices found: ['0x23'] LIGHT: OK light = 250 lux (a normal room gives about 100–500) Fun test: cover the sensor with your hand → the number goes down to almost 0. Shine a phone flashlight on it → it goes way up.

## Step B. The air sensor (BME280)

- Unplug the USB cable.

- Take a medium cable (200 mm).

- Push one end into the free plug on the light sensor.

- Push the other end into either of the two plugs on the air sensor. Keep its other plug free for Step C.

- Plug the USB cable back in and look at the Shell.

WHAT YOU SHOULD SEE Devices found: ['0x23', '0x77'] AIR: OK temp = 23.4 C humidity = 45 % (your numbers will be different) Fun test: breathe gently on the air sensor, or hold it between your fingers. The temperature and humidity should go up. Let go and they slowly go back down.

## Step C. The soil sensor

- Unplug the USB cable.

- Take the soil cable. Look at its two ends. One plug is tiny (the same size as all the other cables). The other plug is a bit bigger.

- Push the tiny end into the free plug on the air sensor.

- Push the bigger end into the white plug on the soil sensor (at the top, the wide end, away from the fork "legs").

- Lay the soil sensor on the table. Do not put it in water or soil yet.

- Plug the USB cable back in and look at the Shell.

WHAT YOU SHOULD SEE Devices found: ['0x23', '0x36', '0x77'] — all three! SOIL: OK moisture = 350 (dry air gives a low number, around 300–400) Fun test: hold the white fork part of the soil sensor between two slightly wet fingers. The moisture number should jump up. Let go and it goes back down.

TIP Soil numbers go from about 200 (very dry) to about 2000 (very wet). These are not percentages. Later we will check what numbers the real orchid pot gives when it is dry and when it was just watered, and use those.

WHAT YOU SHOULD SEE Part 7 is done when LIGHT, AIR and SOIL all say OK at the same time, for at least one minute. Congratulations — the hardware works!

# Part 8. If Something Does Not Work

Do not worry — this happens to every engineer. Always unplug USB first, then check.

| What happens | What to try |
| --- | --- |
| The Mac does not see the Feather at all. | Try a different USB-C cable (it may be charge-only). Try a different USB port on the Mac, without an adapter or hub. |
| The Mac says "you don't have permission", or "Device not configured", when copying. | For CircuitPython: use Way 2 (web installer in Chrome), Part 6.1. For libraries: copy with cp -X -R in Terminal, Part 6.2. |
| All lights on the board went dark and there is no CIRCUITPY drive. | The board is waiting to be installed. It is not broken. Use Way 2, Part 6.1. |
| The web installer is stuck on "Flashing…" but CIRCUITPY is there. | It worked. Close the Chrome tab. |
| ModuleNotFoundError: No module named 'board' | The program was started on the Mac. Do not press Run on the Mac. Save it as code.py on CIRCUITPY (Part 6.4). |
| Thonny shows ImportError or No module named adafruit_... | A library is missing. Check the 5 items in CIRCUITPY → lib (Part 6.2). The bundle version must match the CircuitPython version. |
| Thonny shows nothing in the Shell. | Check that Thonny is set to CircuitPython (generic) and the usbmodem port. Click the red Stop button, then press Reset once on the Feather. |
| LIGHT not found | Push both ends of the long cable in firmly. |
| AIR not found (LIGHT is OK) | Check the medium cable between the light sensor and the air sensor. Push both ends in firmly. |
| SOIL not found (others OK) | Check the soil cable: tiny end in the air sensor, bigger end in the soil sensor. |
| Everything after one board is "not found" | The chain is broken at that board. Try the other plug on that board, or swap in the spare cable. |
| Something gets hot, smells, or smokes | Unplug USB right away. Do not touch the hot part. Tell the grown-up. Take a photo before changing anything. |

STOP AND READ If a board, plug or cable looks different from this guide: stop before forcing anything. Take a clear, close photo of the parts and plugs, and check before turning the power on.

# Part 9. Put Everything in the Greenhouse

Only do this after Part 7 works perfectly on the table.

## Air sensor (BME280)

- Put it inside the greenhouse, near the orchid, in the air (not lying on the soil).

- Keep it away from: water spray, the glass walls, wet soil, and the Feather (the Feather gets a little warm and would make the reading wrong).

## Soil sensor

- Push the fork part gently into the orchid's growing material (bark or moss). Do not force it through the roots — if you feel it hit something hard, pull out and try a slightly different spot.

- Push it in only up to the line where the fork ends. The top part with the plug must stay dry and above the soil.

- Write down the moisture number when the pot is dry and again right after watering. We will use these two numbers later.

## Light sensor (BH1750)

- Put it where it gets the same light as the orchid's leaves, with the little chip facing up toward the light.

- Not under a leaf, not behind the pot, not against the wall, not in a shadow.

## Feather (and later the battery)

- Put the Feather in a small waterproof box, outside the humid greenhouse.

- Protect it from rain, dripping water, watering, and direct sun (sun can overheat it).

- Only the long cable (300 mm) goes from the Feather's box into the greenhouse. If it is too short, ask the grown-up before cutting or changing anything — never cut the cables.

TIP Ask the grown-up to check where water drips when the greenhouse is watered. Make sure no drop can reach the Feather's box or the top of the soil sensor.

# Part 10. Add the Battery (Last!)

GROWN-UP HELPER This part is for the grown-up. LiPo batteries are safe when used correctly, but they can get very hot or catch fire if connected backwards, punctured, or damaged.

- Unplug the USB cable from the Feather.

- Look at the battery's plug and the Feather's battery connector. The red wire must go to the + side printed on the Feather next to the connector. The black wire goes to −.

- If the battery is not from Adafruit, check the colors extra carefully — some other brands wire their plugs the opposite way. If the red wire would end up on −, do not plug it in.

- Plug the battery in gently. Never force it and never pull it out by the wires.

- Check that the Feather starts (its light turns on) and the test program still runs (connect USB to see the Thonny Shell).

- Later, the battery charges through the same USB-C plug. Just plug USB into a charger while the battery is connected.

STOP AND READ Use only a single-cell 3.7 V LiPo battery with a matching 2-pin JST-PH plug. Never connect the battery backwards. Do not charge the battery when it is below freezing (0 °C / 32 °F). On very cold nights, bring it inside to charge. If the battery is puffy (swollen), dented or hot, stop using it and ask how to recycle it safely.

# Part 11. What Happens Next

After this guide, the hardware is finished. The next steps are programming steps, which we will do in a separate guide:

- Teach the Feather to connect to the home Wi-Fi (2.4 GHz).

- Teach the Feather to send its readings to the Raspberry Pi (using the Pi's address from step 4.7).

- Teach the Raspberry Pi to receive and save the readings.

- Make charts on the Raspberry Pi.

- Show the latest readings on the Inky pHAT screen on top of the Pi.

## One reading will look like this

time: 2026-09-22 18:00:00 temperature: 16.2 °C humidity: 71 % soil_moisture: 580 light: 8420 lux battery: 87 %

## What the Raspberry Pi will figure out

| Topic | What the Pi will show |
| --- | --- |
| Temperature | Temperature now; the coldest and warmest of each day; the coldest point of each night; how long it was below 10 °C (too cold!); how long it was in the 13–16 °C "cool night" zone. |
| Cool-night counter | For example "Cool nights: 6 / 14" — in the last 14 nights, 6 were cool enough to help the orchid start flowers. |
| Humidity | Humidity now; the daily average, lowest and highest; times when it was very damp for a long time. |
| Light | Light now; the brightest moment of the day; average light for each hour; a light chart for the whole day. |
| Everything together | Temperature + humidity + light + soil on one chart, so we can see patterns over days and weeks, not just single numbers. |

# Part 12. Final Checklist

## Raspberry Pi

☐  The microSD card is flashed with Raspberry Pi OS (64-bit).

☐  The Pi starts and connects to the home Wi-Fi (2.4 GHz).

☐  The Mac can connect to the Pi with ssh.

☐  The Pi is updated.

☐  The Pi's address is written down.

## Feather and sensors

☐  The Feather shows up on the Mac as CIRCUITPY.

☐  The 5 libraries are in the lib folder.

☐  The test program is saved as code.py on CIRCUITPY and prints to the Thonny Shell.

☐  The chain is: Feather → long cable → light sensor → medium cable → air sensor → soil cable → soil sensor.

☐  Light sensor: found at 0x23, light changes when you cover it.

☐  Air sensor: found at 0x77, temperature and humidity change when you breathe on it.

☐  Soil sensor: found at 0x36, moisture changes when you touch it with wet fingers.

☐  All three say OK at the same time.

## Greenhouse

☐  Sensors are in the right places in the ÅKERBÄR.

☐  Dry and wet soil numbers are written down.

☐  The Feather (and battery) are in a waterproof box, out of rain and direct sun.

☐  The battery was connected only after all USB tests passed — by the grown-up, with red on +.

Great job, builder! The orchid now has its own weather station.